# GW Instek GSP-9300頻譜分析儀拓譜圖功能DEMO

這次的演示是GSP-9300 頻譜分析儀的拓譜圖功能。

這 DEMO KIT是個ASK及FSK的信號產生器。此DEMO KIT有3種預設頻率，3種波特率及可設定輸出為連續或叢發模式。

架設方式：

- 連接DEMO KIT到GSP-9300前面板的USB接口

- 用SMA RF CABLE連接DEMO KIT輸出端到GSP-9300的輸入端。

儀器操作:

- 首先設定DEMO KIT的參數.

- 按Option Control鍵 > Demo Kit[F4]鍵>Demo Kit[F1]鍵*2次,啟動DEMO KIT.

- 按Modulation [F2]鍵, 選擇FSK[F2]鍵

- 按Frequency[F3]鍵,選擇433MHz[F2]鍵

- 按Deviation[F4]鍵,選擇50kHz[F2]鍵

- 按Baudrate[F5]鍵, 選擇波特率為9600[F2]鍵

- 按Output Mode[F6]鍵, 選擇 normal mode.

- DEMO KIT的參數設定結束.開始設定拓譜圖功能觀測FSK的信號.

- 按 Frequency 鍵> Center Freq[F1]鍵>4鍵>3鍵>3鍵> MHz[F2]鍵, 設定中心頻率為433MHz

- 按 Span鍵> Span[F1]鍵 >5鍵>0鍵>0鍵> kHz[F3]鍵, 設定頻段為500kHz

- 在頻譜分析儀上可以觀測到FSK信號.

- 按Display鍵>Windows Setup[F1]鍵>Topographic + Spectrum [F6],啟動Topographic功能

- 拓譜圖以顏色深淺的方式顯示信號出現的機率分布。此功能讓用戶可以直接了解信號隨時間變化的過程，有利於觀測如間歇性的微弱信號或EMI干擾信號。用戶可以運用兩個標記，找出功率相對於頻率和百分比的關係。

- 按Topographic Marker[F1]鍵*2次> 啟動marker使topographic暫停運作。

- 按Ref[F2]>X Axis[F1]>2鍵>2鍵>8鍵>Enter[F1]鍵,設定參考標記頻率reference marker frequency

- 按Y Axis[F2]>1鍵>9鍵>0鍵>Enter[F1]鍵，設定參考標記百分比reference marker percent

- 可以看到參考標記的頻率，百分比及振幅的資料

- 按Return[F7]

- 按Delta[F3]> X Axis[F1]>3鍵>5鍵>0鍵>Enter[F1]鍵,設定差値標記頻率delta marker frequency

- 按Y Axis[F2]>1鍵>8鍵>0鍵>Enter[F1]鍵，設定差値標記百分比delta marker percent

- 可以看到差値標記的頻率，百分比及振幅的資料

- 以上是GSP-9300 拓譜圖功能的介紹.

- 感謝您的收看

# GW Instek GSP-9300 Spectrum Analyzer Topographic Function DEMO

Now, I am going to demonstrate the Topographic function of GSP-9300 spectrum analyzer.

This demo kit is an ASK and FSK signal generator. In this demo kit, there are three preset frequency, three Baud rates and selectable continuous or burst output mode.

First of all, connect demo kit with the USB port located at the front panel of GSP-9300. Next, connect demo kit output terminal with GSP-9300 input terminal with SMA RF cable.

- To begin the operation, first, input demo kit parameters.

- Press option control key, demo kit (F4) key, demo kit (F1) key twice to activate demo kit

- Press modulation (F2) key and select FSK (F2) key

- Press frequency (F3) key and select 433 MHz (F2) key

- Press deviation (F4) key and select frequency deviation of 50kHz (F2) key

- Press Baud rate (F5) key and select baud rate of 9600 (F2) key

- Press output mode (F6) key and select normal mode

- This is the end of demo kit parameters input. Next, set up the Topographic function to observe FSK signals.

- Press frequency key, center freq (F1) key, input 4, 3, 3, MHz (F2) key to set center frequency as 433 MHz.

- Press span key, span (F1) key, input 5, 0, 0, kHz (F3) key to set frequency bandwidth as 500kHz.

- Now, 2FSK signals are shown on the screen.

- Press Display key, windows setup (F1), topographic + Spectrum (F5) to activate topographic function.

- Topographic uses color shade to show the probability distribution of signal appearance. This function allows users to directly understand the process of signal variation according to time changes that is beneficial to observe intermittent feeble signals or electromagnetic interference signals. Users can use two makers to find out the relation of power to frequency and percentage.

- Press Topographic marker (F1) twice to activate marker and stop Topographic operation.

- Press Ref (F2), X Axis (F1), input 2,2,8 and Enter to set reference marker frequency.

- Press Y Axis (F2), input 1,9,0 and Enter to set reference marker time.

- Now, reference marker frequency, reference marker time and amplitude are shown on the screen.

- Press Return (F7)

- Press Delta (F3), X Axis (F1), input 3,5,0 and Enter to set delta marker frequency.

- Press Y Axis (F2), input 1,8,0 and Enter to set delta marker percentage.

- Now, delta marker frequency, percentage and amplitude are shown on the screen.

- This concludes the demonstration of GSP-9300 Topographic function.

- Thank you very much for your interest in GSP-9300.