# GW Instek GSP-9300頻譜分析儀快速掃瞄模式DEMO

這次的演示是GSP-9300 頻譜分析儀的快速掃瞄模式。

架設方式：

- 用BNC CABLE連接GSP-9300 後面板的10MHz參考信號輸出端到GSP-9300的輸入端。

儀器操作:

- 首先按 Frequency 鍵> Center Freq[F1]鍵>1鍵>0鍵>MHz[F2]鍵, 設定中心頻率為10MHz

- 按 Span鍵> Span[F1]鍵 >1鍵>0鍵>MHz[F2]鍵, 設定頻段為10MHz

- 按 Amplitude鍵> Ref Level[F1]鍵>1鍵>0鍵>dBm[F1]鍵, 設定參考準位為10dBm

- 在頻譜分析儀上可以觀測到10MHz信號.

- 目前的設定是正常掃瞄模式,掃瞄時間為17.9ms

- 如果用戶需要加快掃瞄時間,可以按Sweep Mode鍵,掃瞄時間變更為307us.

- 用戶可以用此快速掃瞄模式於捕獲瞬態信號，如藍牙跳頻信號，調諧壓控振盪器，在ISM頻帶的其他干擾信號及其他等。

- 以上是GSP-9300 快速掃瞄模式功能的介紹.

- 感謝您的收看

# GW Instek GSP-9300 Spectrum Analyzer Fast Sweep Mode DEMO

Now, I am going to demonstrate the fast sweep mode of GSP-9300 spectrum analyzer.

First of all, connect 10 MHz reference signal output terminal from the GSP-9300 rear panel and GSP-9300 input terminal with a BNC cable.

- Now, press frequency key, center freq (F1), input 1,0, MHz (F2) to set center frequency as 10MHz.

- Press span key, span (F1), input 1,0, MHz (F2) key to set span as 10MHz.

- Press amplitude key, ref level (F1) key, input 1,0, dBm (F1) key to set reference level as 10 dBm.

- Now, 10 MHz signals can be observed from the screen.

- The current setting is normal sweep mode which has a sweep time of 17.9ms.

- If the faster sweep is required, users can press sweep mode key to change sweep time to 307us.

- Users can use the fast sweep mode to capture transient signals such as Bluetooth frequency hopping signals, tuned oscillator, and other interfering signals in ISM frequency band, etc.

- This concludes the demonstration of GSP-9300 fast sweep mode function.

- Thank you very much for your interest in GSP-9300.